Scapy Lab Documentation 
Scapy is a powerful Python-based network toolkit that allows you to:

- Capture packets

- Generate custom packets

- Analyze network traffic

- Perform network security and testing tasks

- scapy

This command launches the scapy interactive shell. Once executed scapy provides a python prompt where commands like sniff(), summary() and packet inspection are done.

- sniff()

This command starts packet capturing on the default network interface.

What it does:

- Listens continuously for packets flowing through the interface.

- Collects all captured packets into Scapy packet list.

- Continues indefinitely until manually stopped using Ctrl + C

This method is useful for general traffic monitoring when you don’t need interface filtering or packet count limits.

A second terminal is opened to generate live network traffic. This traffic can be then captured and analyzed in the first terminal where scapy is running.

The Command ping google.com is entered into the second terminal and it generated ICMP Echo Request and Echo Reply packets.

The purpose of this command is to:

- To provide a constant stream of packets, which scapy can sniff.

- It helps to test if scapy is capturing packets successfully.

By pressing Ctrl + C in the second terminal and the scapy interactive shell respectively you can stop the continuous ping command and terminate the current scapy sniffing session.

Scapy stores the captured packets in a special variable named _ (underscore), which represents the last returned value.

- mark=_

This command assigns the captured packet list to a variable called mark.

The purpose of this command is to:

- Allow you to store the sniff results

- Make it possible to run analysis commands like .summary(), indexing and filtering.

- mark.summary()

This command displays a one-line summary for each packet captured.

It shows the:

- Packet number

- Protocol (ICMP, TCP, ARP)

- Source and destination addresses

- Basic flags or info

It provides a quick overview of the network activity captured during sniffing

- sniff(iface=”wlan0”)

This command starts sniffing only on the network interface wlan0.

The reason for specifying the interface is that if a system has multiple interfaces(WIFI, Ethernet, virtual bridges), you may want to monitor a specific one.

- ping 10.71.75.192/24

Sends ICMP packets to the device with 10.71.75.192, within the /24 subnet.

It produces local network traffic and allows scapy to capture internal ICMP communications on wlan0.

Opening the browser and typing 10.71.75.192 generates traffic such as:

- ARP requests

- TCP SYN Packets

- HTTP or HTTPS requests

Scapy detects all of these as long as the browser’s requests pass through the sniffed interface.

By pressing Ctrl + C you end the sniff session so the captured data becomes available for analysis.

- mark2=_

This command stores the new set of captured packets into the variable mark2.

- mark2.summary()

This command prints a readable list summarizing all packets captured during this stage.

Filtered Sniffing-Capturing Only ICMP Traffic

In scapy you can filter the type of traffic and specify what type of traffic you want to see. The command sniff(iface=”wlan0”, filter=”icmp”, count=5) is an example of such a command. Here is a simple breakdown of the command:

- iface=”wlan0” – sniff only on the wlan0 or WI-FI interface.

- filter=”icmp” – only capture ICMP packets

- count=5 – automatically stop after capturing 5 ICMP packets.

Filters are useful in reducing noise by ignoring unrelated traffic, capturing relevant packets faster and lowering CPU usage and simplifying analysis.

The ping 10.71.75.192 generates ICMP traffic to match the filter. Once five ICMP packets are detected, the sniffing session automatically ends. The command mark3=_ stores the filtered packet capture in mark3, and the command mark3.summary displays a summary of the five ICMP packets captured which has the following common details visible:

- Echo Request

- Echo Reply

- Source/destination Ips

- Packet numbers

We use the command mark3[3] to display the 4th packet in the sniffed list since indexing starts at 0. This shows full details including:

- Ethernet Layer

- IP header

- ICMP type and code

- TTL

- Checksum

- Payload data

This is useful for deeper analysis or troubleshooting.

The screenshots are stored in the /scapy/screenshots path and each screenshot is labelled according to the output of the command